#### DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700

FAX: 425.867.6972

support@dataio.com

## Subject: Running OPTITEST on PS300 & P100 with OPTIMA programmers

This document describes method to run the “OPTITEST” software on PS300-series, PS200 or PP100 systems with OPTIMA programmers, having either 486-based or Pentium-based Programmer computers (PC104s).

- TOOLS REQUIRED: NONE

- PARTS REQUIRED: NONE

- INSTRUCTIONS:

  - Turn on the system, but do not start AH400 software. If system is on, then completely exit from AH400 software system.

Invoke Windows Explorer and go to I:\MACHINE1\BATCH,

  - Locate the file CNFGETOP.BAT and edit this file by clicking the right mouse button.

  - In this file, there is a line as follows:-

COPY M:\SETTINGS\EXIOADDP.CNF D:\

  - Edit (i.e. change) this line to read:-

COPY C:\EXIOADDP.CNF D:\

  - Do NOT change any other line.

  - Save and exit from the editor.

  - Remove adapters from ALL programming stations.

  - From the AutoView switch box, select Programmer Number-1

  - If OPTIMA software is running, press Esc key to exit and then type D to go to DOS prompt.

  - Go to C drive of Programmer computer (PC104) by typing C:

  - Run OPTITEST from the C: drive  by first going to the C:\> prompt and then typing OPTITEST, that is

C:\>optitest <enter>

  - If there are any messages shown on the screen accept them by hitting Y.

  - Once OPTITEST menu appears, press Esc to exit

  - Repeat step 4.9 to 4.14 for ALL the programmers.

  - Turn off the Programmers Circuit breaker, at the back of the unit, wait few seconds, and then turn on the circuit breaker.

  - Resume normal operation by starting AH400 and loading the required job.

  - Steps 4.10 to 4.14 should be performed if:-

- Programmer computer (PC104) is replaced

- Programmer computer (PC104) changed from 486-based to Pentium-based CPU.

- OPTIMA programmer PCB replaced is replaced

- Programming station added or re-located