10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

Fax: 425.867.6972

pmhelp@data-io.com

SUBJECT: PP100 Package file values that can cause SMAC head problems

| MODELS AFFECTED |  | 2500 |  | 3000 |  | 7000 |  | 7500 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PP100 | X | 9500 |  | 970 |  | T/R |  | ALL |  |

_____________________________________

1.0	PURPOSE:  To inform customer of a condition that can damage the SMAC head of the PP100.

2.0	TOOLS REQUIRED:  NONE

3.0	PARTS REQUIRED:  NONE

4.0	INSTRUCTIONS

  - Go to the C:\AH400_32\package\ directory.

  - Package files will be named, eg. 048TSOP.TXT, S273.TXT etc.

  - Open each package file (using NOTEPAD editor) and verify that:-

- For Record #22 value should be zero (0)

- For Record #46 value should be negative 300 (-300)

  - Any positive value in these lines can cause damage to the SMAC head

Of the PP100.