#### DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

FAX: 425.867.6972

pmhelp@dataio.com

### Subject: PP100 “Invalid Device” Error

Model Affected: PP100

- PURPOSE: To inform customer about an error on the PP100 and how over come it.

2.0	TOOLS REQUIRED: NONE

3.0	PARTS REQUIRED: NONE

- INSTRUCTIONS:  If a job on the PP100 is started with the wrong adapter in the PP100 you will get the error message invalid device code.

  - To get around this error you need to quit AH400.

  - Then cycle the power on the Programmer’s with the correct adapter installed.

  - Next make sure that the PC104’s are on the main Sprint menu.  See Figure 1.

  - Next go the directories M:\Machine1 and go to the sub directories of each programmer and delete the file default.job on each programmer directory.

  - Restart the AH400 job with the correct adapters.

Figure 1