#### DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

FAX: 425.867.6972

pmhelp@dataio.com

### Subject:  PP100 Server file purge

Model Affected: PP100

- PURPOSE:  To inform service personnel about a server option that could effect PP100 operation.

2.0	TOOLS REQUIRED:NONE

3.0	PARTS REQUIRED:NONE

- INSTRUCTIONS: It has been found that some PP100 servers have the purge deleted files option disabled.  Over time these files build up and this could slow down the server traffic.

  - Go to the server monitor.

  - Press Esc to get the Available options.

  - Then go to server parameters.

  - Then go to file system.

  - Then arrow down until you get to Immediate purge of deleted files.  Make sure this on.  If it is off then turn it on.

  - Next go to any of the PC104’s on the Autoview.

  - Next go to DOS.

  - Next go to M:\

  - From the M:\ directory type purge *.* /A

  - This will purge all the prior deleted files from the server.