#### DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

FAX: 425.867.6972

pmhelp@dataio.com

### Subject: OPTIMA programmer keycode / SMP update.

Model Affected: PP100

- PURPOSE:  To inform customer’s how to update the programmer

keycode.

2.0	TOOLS REQUIRED:NONE

3.0	PARTS REQUIRED:NONE

- INSTRUCTIONS: You will receive a keycode from DIO service personnel.

  - Remove all the adapters from the programmers on the PP100.

  - Go to the Autoview and switch to the programmer you are going to update.

  - Go to dos. C:\  then type optitest.

  - You will be prompted to remove the adapters.  Then press any key

  - When the test is finished you will be at the main optitest menu.

  - Press the hidden key 3.

  - Next type in the keycode exactly.

  - If you have typed it properly you will see this menu.

  - If you did not type the code correctly you will see this menu.

  - Try typing the number again.  Remember the codes are serial number specific.

  - Run optitest again and select option 1 Automatic hardware test.

  - Then you will see the new SMP date on the right side.