#### DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

FAX: 425.867.6972

FSE@dataio.com

### Subject: PC104 BIOS Update Procedure

Model Affected: PP100 with Pentium PC104’s and PS300

- PURPOSE: To inform customers how to update the BIOS of the PC104.  Note: this for the Pentium version of the PC104 Only.   486 does not have the PC Card.

2.0	TOOLS REQUIRED: NONE

3.0	SOFTWARE REQUIRED: BIOS.ZIP

- INSTRUCTIONS:

  - Copy the file V2.3.7.zip to a Floppy disk.

  - On the handler computer make a folder on the I:\ Named I:\BIOS.

  - Unzip the file BIOS.zip from the floppy disk into this folder I:\BIOS.

  - You will have 5 files as follows.

P5up.bat

Eraseeep.exe

Devel_tx.rom

Phlash.exe

Platform.bin

  - Go to the PC104 on the Autoview.

  - Then go to DOS.

  - Then change to the C:\ drive.

  - Then type MD P5sev

  - Then type I:\

  - Then type cd BIOS

  - Then type Copy *.* c:\P5sev.

  - You should see all 5 files copied to c:\P5sev.

  - Then type edit C:\config.sys and press the enter key.

  - Then type rem at the beginning of the first 2 lines as shown below.

Config.sys as it looks normally.

DEVICE=c:\dos\HIMEM.sys

DEVICE=c:\dos\EMM386.exe noems i=B000-B7FF x=E000-EFFF

FILES=30

STACKS=18,512

BUFFERS=10

DOS=high,umb

LASTDRIVE=z

REM DEVICE=c:\dos\RAMDRIVE.sys 15000 512 /e

DEVICE=c:\xram\xmsdsk.exe 64000

Config.sys as it will look after editing.

REM DEVICE=c:\dos\HIMEM.sys

REM DEVICE=c:\dos\EMM386.exe noems i=B000-B7FF x=E000-EFFF

FILES=30

STACKS=18,512

BUFFERS=10

DOS=high,umb

LASTDRIVE=z

REM DEVICE=c:\dos\RAMDRIVE.sys 15000 512 /e

DEVICE=c:\xram\xmsdsk.exe 64000

  - Exit and save config.sys.

  - Do control alt and del to reboot the PC104.

You will see that the PC104 did not boot properly press the D key to go to DOS.

  - Then go to the C:\ drive.

  - Type CD P5sev and enter.

DO NOT REMOVE POWER DURING THE REFLASHING PROCESS.

  - From C:\P5sev type P5up and enter.

  - You will see EEPROM is erased please press any key.

  - Press any key to reboot PC104.  Then press F2 quickly to enter the BIOS setup.  If you miss the BIOS setup repeat control, alt and del to reboot the PC104.

  - Go to Boot and load setup defaults.

  - Then exit and say changes.

  - Then press F2 quickly to enter the BIOS setup.  If you miss the BIOS setup repeat control, alt and del to reboot the PC104.

  - Disable Floppy Drive A.

  - Disable USB.

  - Then go to Advanced and disable PS/2 mouse.

  - Then change Large Disk Access Mode to other.

  - Then exit and save changes.

  - Then go back to DOS.

  - Then Type Edit c:\config.sys and press enter.

  - Remove the rem from the first 2 lines as shown below.

DEVICE=c:\dos\HIMEM.sys

DEVICE=c:\dos\EMM386.exe noems i=B000-B7FF x=E000-EFFF

FILES=30

STACKS=18,512

BUFFERS=10

DOS=high,umb

LASTDRIVE=z

REM DEVICE=c:\dos\RAMDRIVE.sys 15000 512 /e

DEVICE=c:\xram\xmsdsk.exe 64000

  - Save the changes and exit editor.

  - Then do a control, Alt and del to reboot the PC104.

  - Then go to DOS.

  - Then go to C:\

  - Then type del P5sev and press enter.  Say Y to remove all files.

  - Then type RD P5sev and press enter.

  - Then repeat this on all PC104’s that are Pentium.

486 does not have this card.