DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

FAX: 425.867.6972

FSE@dataio.com

- Checklist for New Job Creation and Running on PS Series

- Copy and rename known Package and Vision Files using Windows Explorer

Copy and rename a known package file (e.g. C:\AH400_32\package\S509H.txt or PACKAGE TEMPLATE.txt) to the required Package file. (ex: S2023H)

Copy and rename known Reference Vision file (e.g. C:\acuWin95\Test\S509H.prj or TEMPLATE.PRJ ) to the required Reference Vision File. ((ex: S2023H)

- These two will be the “raw” Package & Vision files.

- Create New Job on Programmer Computer – OPTIMA/SPRINT based systems.

- From Data I/O web page find out required Adapter and PROM/PAL for the device. If required, down load and update PROM & PAL on I:\SPRINT\

Select one Programmer Computer (PC104)

- Install required Adapter on the selected Programmer

Enter Job/Task Name (ex: BIOS7JOB)

Enter Adapter Name as Package Name (ex: S2023H)

Enter Marking File name, if required (ex. BIOS7)

Select PROM or PAL and load Data file -note Checksum

- OR -

- Create Task / Job form TaskLink for Windows

- Using Package and Vision file names from previous step.

- Load newly created Job/Task – for systems without “TaskLink for Windows”

Start AH400 software and load newly created Job/Task (ex: BIOS7JOB).

- OR -

- Run AH400 by hitting [Run] within TaskLink

If required, pre-select programmer(s).

- Install / Setup Input & Output Media, Adapters and adjust Socket Actuator

Bring PnP (Pick-and-Place) head over one programmer & center over socket adapter.

Adjust Socket Opener (with main air turned off) and its ribs so that socket is fully open when Socket Opener comes down

Create Reference Vision file on the Handler Computer, from “raw” file.

Within Gantry screen, Go To Vision and "Teach" such that X,Y error <= 0.003 and Verification > 95

- Create Package file on the Handler Computer, from “raw” file.

Teach all locations - Input media, programmers, Output media, marking station etc.

- Create Marking file on the Laser Computer – Optional -

Invoke WinMark-Pro software and load an existing marking file and "Save As" required file (ex: BIOS7)

Adjust device/text placement to desired location on device body, within Gantry or AH400 System /Shuttle/Options screen

Save marking file, quit (close) WinMark-Pro and invoke "Serial Marking Interface" software.

- Install / Setup Input & Output Media, devices and Run Job

Press Run to start job.