DATA I/O Corporation

- 10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

FAX: 425.867.6972

support@dataio.com

### Subject: PP100 / PS-Series Cognex acuWin Vision System Setup

This document provides guidelines on setting up the acuWin vision system’s COGNEX ISA Vision card on PP100 and PS-Series. This may be required if there are conflicts between resources used by Vision card and other cards on handler computer especially network interface (NIC) cards used by Novell NetWare.

Typical symptoms would be errors as soon as AH400 is started and then acuWin software will fail to open, giving following error messages: -

com_init: Host memory conflict

sys_init: TMG load error

com_pixget failed

Abnormal program termination

To correct this: -

- Power down the handler computer

- Change VisionCard's I/O address to 0x01C8 by locating the DIP switches near the bottom of the card and set them to:-

SW-1 ON

SW-2 OFF

Sw-3 ON

- Turn on the handler computer and navigate to C:\acuwin95 directory

- Locate setup95.exe and run it.

- On acuWin setup screen, under “VisionCard Configuration”

Set MemBase to D400

Set IO Base to 1C8

- Click on [Configure] to end Setup.

To check, run ACUWIN32.EXE, enable “Continuos Acquire” and place hand above camera. If picture appears, then acuWin is working normally

Note: acuWin will run only if the cards' IO addresses and the addresses selected in the Setup program are the same.