10525 Willows Road NE

Redmond, WA 98052

Phone: 1.425.881.6444

Phone: 1.800.247.5700 (press 2)

Fax: 425.867.6972

pmhelp@data-io.com

### SUBJECT:  PP100 Master Device Usage

| MODELS AFFECTED |  | 2500 |  | 3000 |  | 7000 |  | 7500 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PP100 | X | 9500 |  | 970 |  | T/R |  | ALL |  |

____________________________________

1.0	PURPOSE:  To inform customer’s about using master devices on the PP100.

2.0	TOOLS REQUIRED:  NONE

3.0	PARTS REQUIRED:  NONE

- INSTRUCTIONS:  The PP100 can not use a master device as a data source.  You will need to create a data file from RAM.

  - Go to the programmer interface.  Switch B on the Autoview.

  - Install the correct adapter for the device to the program that you are connected to.

  - Press any key to escape the Sprint menu.

  - Then enter D for DOS.  At the D:\  prompt enter PROM or PAL depending on which type of device you have.

  - Select the device that you have.  Example:  STM 27C801

  - Insert the device manually.  Then select R = Read device This will load the data into the RAM.

  - If you have a PROM that was from another programmer you may need to change the RAM fill to FF to get the correct checksum.

  - To change the RAM fill press O for options.  Near the bottom the screen you will see a line that says  Default Buffer contents      FF Hex or Default Buffer contents   00 Hex.  Use the space bar to change.

  - Once you have the correct check sum select W = Write HEX or BINARY disk file.  You will be prompted for file type INTEL-HEX [H], SPACE-HEX [S] or BINARY [B]  Note Type S does not work.

  - When prompted for the file name and location, type  I:\PRGDATA.

  - To test this file select I = Input file from disk to memory.  Select this file and see if it loads correctly.

  - If it loads and you get the correct check sum you are done.